TCM203

产品简介 General Information

- TCM203是一款适用于阳离子固化的双官能团氧杂环丁烷单体。它能提高体系固化速度，降低收缩率，同时提供良好的耐化学性和极强的物理强度。

- TCM203 is a bifunctional oxetane suitable for cationic curing. It can improve curing speed, reduce the shrinkage, provide good chemical resistance and excellent physical strength.

产品信息 Product Information

- 化学名称 Chemical Name：1,4-双[(3-乙基-3-氧杂环丁基甲氧基)甲基]苯 1,4-Bis[(3-ethyl-3-oxetanylmethoxy) methyl] benzene

- 分子量 Molecular Weight：334.45

- CAS No：142627-97-2

技术参数 Technical Parameters

- 外观 Appearance：非常粘稠黄色液体至固体 Very viscous yellow liquid to solid

- 水分 Water Content：≤0.15%

产品优势 Benefits

- 高反应活性 High reactivity

- 良好的耐化学性和极强的物理强度 Good chemical resistance and excellent physical strength

应用 Applications

- TCM203常作为单体用于电子材料封装和特种胶粘剂。使用前请预加热至50~60ºC以降低粘度。

- TCM203 is often used as a monomer for electronic material packaging and special adhesives. Please preheat to 50~60 ºC before use to reduce viscosity.

存储及包装 Storage & Packaging

- 保存在密闭容器中储存在阴凉、干燥、阴暗、通风的区域。开封后会逐渐吸潮失效。自生产日期开始，6个月内有效。包装：20kg/桶。

- Keep in a tightly closed container and store in a cool, dry, dark, ventilated area. After opening, it will gradually absorb moisture and become invalid. Effective from the date of production, within 6 months. Packaging: 20kg/pail.

THE END